Mark O’Bryan

Architecture, School of Architecture, College of Design at the University of Kentucky, Lexington, KY, USA

Visualization Principles for the Beginning Design Studio

Introduction

“The concept of visualization, as set forth in this series, is extremely important; it is a subjective, creative approach and must not be confused with a photometric ‘tone-reproduction’ approach, systematically defined.”

- Ansel Adams, The Negative

Teaching the creative process, whether it be photography or architecture, means preparing students to visualize. However, architecture design students have no construction experience, which makes it difficult for them to visualize a building solution. Inexperienced with architectural building construction, their judgment of scale, structural forces and other priorities are impaired. Studying existing examples is also a limitation because of an inability to see inside walls. Additionally, while architecture books have many types of representations they often lack details of construction. This paper and conference presentation explores the ways in which the author has attempted to bridge this gap between architectural construction and visualization. Specific references are made to architectural principles taught in the beginning design studios led by the author between 2010 and 2016.

Figure 1, for example, illustrates student house designs from 2011 to 2016.  Each progressive year illustrates greater success with emphasis on details utilizing a teaching approach centered on visualization. The composite demonstrates a reduction in project size and increase in technical understanding of architectural orders of construction, which leads to greater levels of overall detail in the finished projects. Future research, however, is necessary for future development of assessment tools to aid in student learning. For instance, how can projects that are understood more completely lead to greater student efficacy and confidence? How might this type of learning as evidenced by student work samples be better measured, specifically how can student learning be measured over time?

World famous photographer Ansel Adams wrote in his seminal book, The Negative, that “visualization” is the subjective formation of a mental picture of the photograph prior to the shutter’s release. The photographer is able to scan a scene and simultaneously use the imagination to visualize the final product. For the architect, visualization is much the same. It is the ability to imagine a building prior to construction. However, the ability to envisage a design before it exists requires knowledge of detailed architectural construction. A student may visualize a design, however that does not necessitate that it is connected with a detailed image of a finished building. This is because architectural construction is complex (Fig. 02) and details need to be understood as systematic. The student of architecture needs to understand details in context to the whole structure.

This author focuses on a small project involving very specific architectural elements (kit of parts and parameters) and develops thinking outward. Assumptions are made about general cost and scope with small cardboard sketches. Specific construction details of all connections are explained repeatedly and students are expected to know everything about the final design to the point of actual construction drawings. By making a small design in a fully complete set of representations, a better understanding of architectural construction is learned. This results in a better intuitive anticipation of a positive result for future design projects.

Focusing on a design process that utilizes few elements increases the likelihood for success, which in turn builds confidence. Confidence is crucial to the assessment of a familiar problem and utilizing knowledge to address it. While this method of teaching may appear obvious, it is not practiced in schools of architecture. Teaching architectural construction is the most difficult thing to do. How do we teach pre-visualization to architecture students? How do we instill success and confidence? These questions are addressed in the paper and presentation. The beginning student must be taught the standards of design and this means the basics (Fig. 03).

The Teaching Process

Students in the early stages need to try various approaches and overthink, or be overly self-critical of, beginning arrangements. At times, rearranging a set of simple but well detailed form relationships can result in an interesting design if the attention of both the professor and the student is focused. This is not typical because the average architecture professor and student lack direction detail focus. The author emphasizes the balance between the intuitive and systematic reasoning of students by integrating hand making and computational processes.

In general, the key successes to the studio are always that students know how to build their project as architecture and solve the complex space arrangements with purpose. From experience, this goal is impossibly difficult with beginners because they have neither construction experience nor knowledge of dimension standards. This is why at the earliest point in the design curriculum students are advised to think holistically. Design requires knowledge of how spaces are normally used and how work and living in space translates into standardized configurations. Additionally, it requires consideration of how the actual cost of construction places pressure on the efficient organization of space. Standards also contribute to mechanics, which then get modified by other factors such as structural repetition and planning by zones according to simple regulating geometry. Plan generators are part of design learning. This learning is visualized by repeated study and analysis of existing standards. Students must learn to visualize scale incongruous with standards of hierarchy in plan design. One of the biggest teaching errors along with the absence of systematic detailing is the lack instruction on planning. Limiting area options by dictating a planning grid is recommended. The students’ gap in knowledge and experience is narrowed by repeated and rigorous analysis and corrections (Fig. 04).

The beginning design students have been exposed to media rich environments, but have little experience in rigorous hand drawing or construction. Combining hand drawing with digital media encourages students to think and see in terms of architectural orders of construction. Drawing demonstrations use overlay trace and overlay digitally constructed print media to help visually explain the rule systems and architectural orders of construction. Students are introduced to full-scale detail drawing and are expected to draw complex construction sections and detailed assemblies by the end of the year. A Structural and Material Concepts companion course with a BIM software requirement balances speed and accuracy with rules and design concepts. Another innovation is a method of learning as teaching where students are given teaching assignments involving details of visualization.

Using a combination of representations displayed in the studio allows the student to see very high standards of quality. Hand drawing assignments with balanced attention on concept combined with speed and precision focuses the student’s visual awareness on “seeing.” Later on when students are working in virtual digital BIMM environments they are shown how hand drawing trace overlays on top of digital output can focus attention for design edits and detail development. The visual accuracy inherent in BIMM environment with the return to hand editing makes fluid the act of design and creates a more complete picture of the developing context for the student to see and make improvements to the design. This in turn improves seeing and thinking in this curriculum. Criteria and evidence from past studios demonstrates substantial improvement in student representations.

Looking back at examples from previous years before 2011 until 2013, student work rarely achieved the goal of architectural construction knowledge complete with material finishes. In hindsight, the reason for this lack of finish was in part due to an over ambitious multi-project schedule and, as previously mentioned, lack of construction visual guidance. Additionally, there are five principles of design that have helped this author in teaching visualization in the beginning design studio. These principles are elegance, performance-based function, few-arbitrary and adjustable parts, object self-explanation, and symbolism. These five principles, which are discussed further in the next section of this paper, as well as construction require more time in the schedule.

To compensate for this time required, the author has reduced the project to a smaller size and the number of projects to one. Also, more time is devoted in studio to technical development and finish.  Reducing the schedule to revolve around a single project and intensifying the detail requirements has the positive result of maximizing student focus. The building is small enough to visualize all of the details of construction at a large enough scale where they can be seen together in a single printed drawing. This is best practice for the obvious reason that both students and professor can see (visualize) if detail is absent, incorrect and or significantly contributing to an intended artistic effect. The detail representation requirements allow for all to diagnose problems at every level of design. This is highly beneficial because it keeps everyone on his or her toes and ensures complete understanding (Fig. 05).

Additionally, various methodologies have been used when teaching architecture and several have been integrated into the author’s first and second year studio curriculum. For example, the statement of the assumptions. Defined assumptions are important for many reasons. Students need to recognize and understand what they will learn during the course and, more importantly, why they need to learn the objectives set forth. This statement of assumptions should be done in simple, clear terms. For example, the author might tell students they will need to know the thicknesses of walls and where the studs are in the wall. They must also know when it is important to show the studs in the plan and when it is unnecessary. This kind of teaching might appear trivial or nit picky. However, if the walls have stud spaced columns that connect to space beams then this information is critical to the architecture. The same can be said if special modular finish paneling is used. It is a simple, yet complex, design concept the student must understand (Fig. 06).

In the beginning, students must also be introduced to the relationships between structural components and finished building components. They must not shirk the responsibility to prove and verify architecture constructability. The author uses a case study method to explore the successes and failures of a range of example projects, both commonplace comparisons and pedigree based types. One example of verification is to evaluate beam-ends in the façade. From teaching a studio course, the author has seen the best students regularly fail to correctly show the end of a beam and the side of a beam in critical section elevation drawings. So then details are part of a formative pedagogy – a sequence of feedback loops that has in turn informed subsequent lesson plans.

The beginning design process must stem from “visualization” in order to result in an architecturally constructed final production. The process looks at the uses of quick sketch models by hand and the sketching within actual and virtual environments to test thematic alternatives. It uses ideas such as the architectural orders of construction – design thinking, systems thinking, scope thinking, containment thinking, inspired detail thinking, typical detail thinking, and “improvisational” thinking (Fig. 07) to develop sketches into buildable projects.

Principles of Architecture and Teaching Perspectives

There are principles in the field of architecture that are key to understanding its meaning. These “good principles” include elegance, performance-based function, few-arbitrary and adjustable parts, object self-explanation, and symbolism. Good, orderly principles are mutually dependent and students must learn to practice vigilance in the enforcement of architectural orders of construction. These five orderly principles can be broken into two main categories: (1) Elegance, Performance with Few Arbitrary or Adjustable Parts and (2) Self-Explanation and Symbolic. The first places emphasis on editing and simplification - uniformity and complexity together with usefulness and service. The second refers to ideas about identity and meaning. The object appears to celebrate and represent purpose, cultural value and/or some specific and important memory memorial. The combination of the two categories is an example of art of building, or in other words, architecture. These five principles are not to be regarded as a recipe, but rather as visual guidelines.

In teaching the beginning design studio, the cultural meaning of good principles is discussed at each stage of the design process. The context of design involves practice of architecture and the licensure process. It is important that professors of architecture anticipate the practice of architecture by discussing ideas like health, safety, and welfare. It is essential to explain to students why standards and conforming principles of design ensure culture and the public good. These should be the main guiding principles behind practice. Beyond the scope of this paper, this is a discussion that is crucial to architecture as standards of conformity, as applied to building, are essential to defining what architects actually do and their value in society. It is important students learn these conforming standards and it is best they master these as early as possible. Additionally, demonstrating “goodness” using specific examples of student work, the author, in a studio setting, reciprocally aligns the representations with thinking and thinking through models.

Teaching perspectives have changed over the past several decades with changes in computing technology. These evolutions parallel critical changes in design education that the author often uses as a mode of pedagogical inquiry. Advanced computing is now affordable. This change, particularly with students having free software to use, allows much more precision in details and in representation of various options to test and resolve. The available software also can lead to further expansion of the form language. This expansion is both a blessing and a curse in that it becomes harder and harder to find and ensure that there is enough common language to relate projects to each other in context. The idea of context, urban context, and community are still very real considerations for future architects. The author discusses these ideas with students and hopes that in their future they will be able to offer value-added design.

In conclusion, with the advances in computing, particularly in rendering that allows students to produce professional level products, comes the benefit that students are able to produce professional quality drawings. They are not limited by the tools available. Now students are able to think and utilize computing with a competitive advantage, to “previsualize.” This author places value on teaching hand drawing detailing in combination with digitalization. The mixture provides students with the skills and knowledge needed for thoughtful design and visualization. The use of digital tools, hand drawing, research precedents, and the creation of both physical and digital models inform the work habits and time management of students as they generate their presentations for juried reviews (Fig. 08 and 09).

Bibliography

Adams, Ansel. The Negative. New York: Little, Brown and Company. 1959.

Figures and tables

Fig. 02. Composite image of design process including graphic reference and architectonic standards are necessary to combine common sense with formal and symbolic types. Beginners and experienced designers always reference standards. All design is basic design using a trial and error process.

Source: Mark O’Bryan

Fig. 01. Composite of student final drawings from 2016 demonstrates a greater emphasis on details. Future research will need to be undertaken to develop more precise assessment tools for student learning.

Source: Mark O’Bryan

Fig. 03. (above) Relatively small full-scale architectural models are derived in complex 2D and 3D digital drawings using Autocad and Rhino. Rhino, Revit and Maxwell are used to learn all the necessary drawing, modeling and rendering conventions.

Source: Mark O’Bryan

Fig. 04. (left) Students design a small house, which receives a full technical review. Full technical reviews are hardly ever practices in education. The full technical review is taught at great length in order to balance and better focus the broader design intentions. The plan is the generator.

Source: Mark O’Bryan

Fig. 05. (left) and Fig. 06. (bottom left) Section and perspective are keys to representation of space form. All of the complex relationships developed in the project are represented in these views. Material qualities are tested and evaluated. Full technical reviews are essential for showing completion of a design.

Source: Mark O’Bryan

Fig. 07. (above) Design reviews teach  oral communication. Students need to design the best use of words combined with images.

Source: Mark O’Bryan

Fig. 08. (above left) and Fig. 09. (above right) Final presentation boards teach total graphic communication of a project.

Source: Mark O’Bryan

Adams, Ansel. The Negative. New York: Little, Brown and Company. 1959.