Davide Schaumann a, Yehuda E. Kalay b, Seung Wan Hong c

a, b Faculty of Architecture & Town Planning, Technion – Israel Institute of Technology, Haifa, Israel, c Department of Architecture, Inha University, Incheon, Republic of Korea

A Form-Function-Use (FFU) model to simulate human behavior in built environments

Abstract

Current building modeling tools, including popular BIM (Building Information Modeling) systems, provide a static representation of buildings, which neglects dynamic aspects concerning how people will use a building after it has been realized. Consequently, during the design process, architects are required to fill this informational gap with their own knowledge and assumptions, based on precedents drawn from other projects, which may apply only partially to new design contexts. To address this issue, we propose to augment digital building models with information pertaining to their function, and use. Function consists in a set of attributes that reflect how a space can be used, by whom, and under which circumstances. Use is the result of the dynamic interaction between a building, its expected users, and their goal-oriented activities. In this work, we propose a simulation framework able to represent form, function and use. We demonstrate the method in the context of hospital environments, where the dynamic relationship between human behavior and the built environment are difficult to be anticipated during the design phase.

Introduction

Prediction and evaluation of the future expected performance of buildings is of cardinal importance for architects and their clients. But because of their size and cost of construction, buildings cannot be prototyped––they are a prototype of themselves. If the architect makes a mistake that is not recognized during design time, it can only be corrected at great cost, or not at all. To overcome this problem, architects have been using different kind of representational means to predict and evaluate various building performances. They could thus contemplate alternative design solutions, involve more people in the decision-making process, and evaluate the desirability of potential outcomes.

The advent of Computer-Aided Design (CAD) in the 1960s enabled the development of digital building representations that support geometric modeling and photo realistic image rendering. Other types of prediction and evaluation, such as structural analysis and energy prediction, required the use of multiple, separate external databases to augment the model and avail it to analysis programs. The emergence of BIM (Building Information Modeling) at the end of the 20th Century added much needed non-graphical data to the formerly mostly geometric representation tools, supporting in an integrated way different kind of analyses, including cost, energy, light, acoustics, and structure.

Despite these advancements, current building models fail to represent how a building will actually be used by its occupants prior to its construction and occupation. Instead, to address this important type of performance, architects mostly rely on their personal knowledge, which is often biased, and on accumulated experiences gleaned from prior case studies and post-occupancy evaluations (POE), documented in the form of codes and regulations. These methods ignore the dynamic and stochastic nature of human activities and the site-specificity of building projects: a building that performed well for one community of users, at a given location, may not work well for a different community of users, at a different time and location.

We propose a more comprehensive representation model that can support prediction and evaluation of how people use buildings. Such representation considers not only the form of the building, but also its function and its use. We demonstrate the proposed approach in the context of hospital environments. Designing hospitals is a complicated task because of the wide range of users and the variety of functions that are carried out in the same location. Still, hospitals also demonstrate a high level of formalization, which is advantageous for the purposes of our research: it provides a comprehensive and agreed-upon set of use-processes on which to test the model.

Relevant studies

The importance of architectural models has been recognized for more than 2,500 years. First came physical scale models, which were often crude and imprecise approximations of the actual building. The invention of scale drawings and perspective rendering in the 15th Century, as means for design conception, representation, and communication, allowed architects to conceive and represent buildings on paper before they were committed to timber and stone. They could thus contemplate alternatives design solutions, involve more people in the decision-making process, and evaluate the desirability of potential outcomes.

The advent of CAD in the 1960s allowed architects to further evolve the representational means at their disposal, mostly in terms of photo-realism and control over detail. The emergence of BIM the dawn of the 21st Century added non-graphical data to the formerly mostly geometric representation. In BIM systems, buildings are modelled as an aggregation of objects, describing various components such as beams, slabs, walls, windows, doors, etc. Such models have been primarily used to produce building analyses in terms of cost, structure, acoustics, and illuminance, which represent the relationships of the building with different physical phenomena such as temperature, light, wind, and gravity. In this work, we focus on a different kind of analysis, one that investigates how people use buildings. Analyses of this kind require not only a model of the building and its physical parts, but also of the semantics of buildings, their intended users, and a series of activities they will perform.

To address this issue, research efforts have been directed to expand building models with information about the activities performed in space,,. A configurational approach has been proposed by Hillier & Hanson to analyze people movement in spaces using a graph-based representation that measures spatial relations and connectivity. Multi-agent systems have also been developed to simulate specific aspects of human behavior, such as pedestrian movement, egress situations, activity schedules in office buildings, and scheduled and unscheduled activities in hospital buildings. The latter model is centered on the concept of Events—computational entities that direct the behavior of multiple agents in space.

We propose an advanced building representation that supports multi-agent simulations like the one proposed in Schaumann et al.0. The model encodes information about Form, Function, and Use to support the simulation and analysis of how people use buildings. The paper expands on these three components, which have been previously introduced by Kalay et al., and illustrates an application of the method in the context of hospital environments.

The Form-Function-Use Model

In this section, we describe the Form-Function-Use model, and its components. Tab.01 summarizes key features of the model.

Table 01. Description of key model features

|  | Form | Function | Use |
| --- | --- | --- | --- |
| Questions | What does it look like? What is it made of? | What is it? What does it mean? How it can be used? | Who uses it? How is it used? When is it used? For how long? |
| Model Attributes | Geometry Topology Materials | Semantics Affordances | Detect Users & Activities in space Define current use |
| Dependency | Physical qualities | Social agreement Building program | Building operation Spatio-temporal conditions Social factors |

Form

In the proposed approach, Form describes the characteristics of a building and its parts, including its shape, position in space and materials. It represents architectural elements that are both physical (e.g. walls, windows, columns, floors, and so on) and non-physical (e.g. rooms, courtyards, corridors), including the spatial/topological relationships between them. Form can be modeled using existing BIM tools, which provide a systematic digital representation of objects, and the relationship among them. Fig.01 provides an example of such model.

Fig. 01. Space model with form attributes

Function

Function is a set of conceptual attributes that add meaning (semantics) to form characteristics. It describes how an object can be used, for what purpose, and within which context11. In architectural design, function is often represented as the building programme, which reflects the goals and desires of a client in terms of building specifications. Function is independent on Form, because different ‘forms’ can serve the same function: consider for instance a design competition, where architects compete to generate the best form that will respond to a specific building programme. Conversely, the same ‘form’ can have different functions: historic buildings maintain the same form over decades while changing their function.

BIM models do not explicitly represent function. Instead, they rely on the assumption that the professionals who use the model all share a common understanding, and therefore will interpret the meaning of the represented objects in similar ways. This assumption is fundamentally flawed, as building stakeholders often include professionals from different domains. For example, in the case of a hospital, they include architects, doctors, nurses, administrators, lawyers, lay people, etc., each with a different view of the world, and a different interpretation of the building model.

To overcome this representational deficiency, we model function in terms of particular semantic values associated with each space, which describes how it can be used, and under what conditions (e.g. patient room, nurse station, corridor, etc.). This semantic information assigns to each space a list of affordances, describing possible context-dependent uses. For instance, a “patient room” may be associated with the following affordances: “clinic”, “social”, “private”, “assistance”, which are activated based on the users present in the room, and the activities they perform. The activation of a specific affordance determines how a space it is currently used. It also stipulates which other affordances are supported in the same space, therefore allowing or inhibiting other behaviors. For instance, a patient room that is used as clinic during a doctor’s visit, prevents visitors from using the same space for social purposes, such as for talking with the patient. Fig.02 provides an example of function attributes that can be associated to a space model.

Fig. 02. Examples of function attributes

Use

Use is a property that adds time-based, socio-cultural information to the represented artefact. It emerges from the interaction between the object and its user. The use model requires an advanced description of the building users, the activities they perform, the spaces they inhabit, and the events they are involved in. User models describe an occupant’s functional role (e.g. doctor, nurse, visitor, secretary), characteristics (age, gender), dynamic status (tiredness, stress), and preferences. Activities describe a set of scheduled procedures (e.g. a doctors’ round) or unscheduled procedures (e.g. the social interactions between patients and visitors). Spaces encode information about their form, function, and current use, including who are the actors involved, what do they do, and what is the current “use” of the space and the available affordances. Events are computational entities that coordinate the convergence of users in space to perform specific activities. Different from Agent-based models, where agents (users) are equipped with independent decision-making abilities, Events dictate what agents should do in specific circumstances. Hence, they afford a higher-level view than an individual agent has, and can therefore coherently coordinate the actions of multiple agents that depend on one another. For example, a “patient check” event ensures that a nurse, a doctor, and a patient convene at a specific location (e.g., the patient’s room) at the same time, and will commence only after they have all arrived. Another Event may direct a group of visitors to talk with a patient.

While performing, Events monitor the current state of the world (e.g. the state of the users involved, or the space in which they are located) to determine whether a necessary set of conditions prevail. For instance, an Event coordinating the social interactions between a visitor and a patient needs to monitor that the space in which the users are located (in this case a patient room) currently supports “social” activities. This would be the case, for instance, if no doctors are currently located in the room. The eventual presence of a group of doctors and nurses would cause the space semantics to become “clinic”, hindering the performing of any “social” activities, including the social interactions between patients and visitors. In this case, the social interaction Event will include provisions what to do, such as directing the visitors to move to the corridor and wait for the conclusion of the medical treatment.

Fig.03 illustrates an example of such scenario: in part (a), a group of doctors and nurses performing a doctors’ round enter a patient room, while a group of visitors socialize with a patient in the adjacent room; in part (b) the medical staff enters the room where the visitors are located, causing a change in the room semantics, which in turn causes the visitors to leave and wait in the corridor.

Fig. 03. Example of the dynamic change in space use

Case study

Fig.04 displays a simulation snapshot of an internal medicine ward “in use” prior to its construction. It is populated by virtual doctors, nurses, patients, visitors, and technicians, performing a variety of activities. The main activities performed involve a doctors’ round, visitors socializing with patients, and a medicine administration procedure. The movement of each actor is traced using colored lines (red for doctors, orange for nurses, blue for visitors).

The simulation reveals the mutual impact of people and a built environment. It shows how the design of a built environment affects the way it is used by its inhabitants. For instance, the lack of appropriate spaces for visitors to wait may determine the level of congestion in the ward corridors, causing delays in providing medical treatment to patients. The provision of a dayroom, instead, where visitors may spend time with patients, may alleviate that situation.

Fig. 04. Simulation of a building in use

Discussion

The proposed modelling approach enables the simulation of how people use buildings prior to their construction and occupation. Different kinds of analyses may be performed, such as measuring walking distances, the amount of time a given space is used, for what purpose, by whom, etc. Eventually, multiple design alternatives could be compared and evaluated against such metrics. We argue that the proposed system holds the potential for generating useful information that can be used in architectural design to evaluate and compare different design options as far as their use is concerned.

Acknowledgments

This research was made possible by the generous support of the European Research Council grant (FP-7 ADG 340753).

Kalay, Yehuda E. Architecture’s New Media: Principles, Theories, and Methods of Computer-Aided Design. MIT Press. 2004.

Simeone, Davide; Schaumann, Davide; Kalay, Yehuda E.; Carrara, Gianfranco. “Adding users’ dimension to BIM”. Milan: EAEA conference. 2013. p. 483-490.

Eastman, Charles M. & Siabiris, Anastassios. “A generic building product model incorporating building type information”. Automation in Construction. no. 4. Vol. 4. 1995. p. 283–304.

Ekholm, A. Modelling of user activities in building design. Architectural Information Management. Helsinki: ECAADE. 2001. p. 67-72.

Kim, Tae Wan; Rajagopal, Ram; Fischer, Martin; Kam, Calvin. “A knowledge-based framework for automated space-use analysis.” Automation in Construction. vol 32. 2013. p. 165-176.

Hillier, Bill; Hanson, Julienne The Social Logic of Space. Cambridge University Press. 1984.

Kapadia, Mubbasir, & Badler, Norman I. "Navigation and steering for autonomous virtual humans." Wiley Interdisciplinary Reviews: Cognitive Science. no. 4. vol. 3. 2013. p. 263-272.

Chu, Mei Ling; Parigi, Paolo; Law, Kincho; Latombe, Jean Claude. “SAFEgress: A Flexible Platform to Study the Effect of Human and Social Behaviors on Egress Performance.” Tampa: Proceedings of the SimAUD conference. 2014. p. 35-42.

Goldstein, Rhys; Tessier, Alex; Khan, Azam. 2011. “Space layout in occupant behavior simulation.” Proceedings of IBPSA-AIRAH Building Simulation Conference. p. 1073–1080.

Schaumann, Davide; Kalay, Yehuda E.; Hong, Seung Wan; Simeone, Davide Simulating Human Behavior in not-yet Built Environments by means of Event-based Narratives. Washington DC: Proceedings of the SimAUD conference. 2015. p. 7-14.

Kalay, Yehuda E.; Schaumann, Davide; Hong, Seung Wan; Simeone, Davide. Beyond BIM: next-generation building information modeling, to support form, function, and use of buildings. In Kensek, Karen, and Noble, Douglas (eds.) Building information modeling: BIM in current and future practice. John Wiley & Sons. 2014.

Brodeschi, Michal; Putievsky Pilosof, Nirit; Kalay, Yehuda E. “The definition of semantic of spaces in virtual built environments oriented to BIM implementation”. Celani, Gabriela; Sperling, David M.; Franco, Juarez M. S.  (eds.). The next city. San Paulo: CAAD Futures. 2015. p. 331-346.

Norman, Donald A. "Affordance, conventions, and design." Interactions. no. 6. vol. 3. 1999. p. 38-43.

Gibson, James Jerome. The Ecological Approach to Visual Perception. Lawrence Erlbaum Assoc. Hillsdale, New Jersey. 1979.

Sopher, Hadas; Schaumann, Davide; Kalay Yehuda E. “Simulating Human Behavior in (un)Built Environments - Using an Actor Profiling Method”. Engineering and Technology International Journal of Computer, Electrical, Automation, Control and Information Engineering. no. 10. vol. 12. 2016.

Schaumann, Davide; Kalay, Yehuda E.; Hong, Seung Wan; Simeone, Davide Simulating Human Behavior in not-yet Built Environments by means of Event-based Narratives. Washington DC: Proceedings of the SimAUD conference. 2015. p. 7-14.