Cameron Purdie

architecture4u, Glasgow, Scotland.

Interpreting Visualisations

Introduction

Public Inquiries are now common place to permit the review of sensitive development proposals in increasingly scarce landscape settings. Much of the review time centres on the visual impact since rural populations have to live cheek by jowl with the resultant structure. Recent decades have seen great advances in the quality and realism of the visualisations prepared for assessment. Equally, industry guidance and standards have developed methodologies and procedures to ensure as comprehensive an assessment is undertaken as possible. However, photomontage based visualisations appear to be still prone to error and qualitative written judgements about the likely visual impact associated still provide a contentious forum at Inquiries. Taken together these factors continue to mask a deeper understanding in visual assessment and can therefore lead to poor interpretation of images which in turn has the potential to mis-inform and regretfully misguide decision makers.

Difficulties

In 2012, I was asked to assist a community group in the visual assessment of a proposed extension to the Black Law wind turbine development (Phase 2) in the central belt of Scotland. The development is now complete and I have been able to revisit the visual impact evidence originally submitted to the Public Inquiry and compare it with the reality of construction over recent months. Two key issues have emerged.

Firstly, the visibility of the Phase 2 turbines is greater than predicted in some of the original visualisations. Largely, this is due to particular viewpoint locations which do not show the original wind turbines in the view and therefore lack any helpful positional control points. These views tend to be in the lee of hillsides and unfortunately in populated areas. The more open and long distance views do not suffer from the lack of positional control features and appear to be fairly accurately represented in the developer’s photomontages. Accurate visibility prediction remains a difficulty, especially in the more sensitive areas of the landscape where the topography is more shaped and the development is likely to be in closer proximity to population centres. Further examination of accuracy in photomontages is a separate and ongoing research exercise and not pursued in this paper.

Secondly, the qualitative visual impact judgements for the most contentious views have been re-examined and are presented here for discussion. (Fig.01) shows a view of the recently constructed turbines in the foreground set against the original turbines in the background. This view was assessed by the developer’s landscape and visual consultant at the time of the Inquiry as having ‘scale similarities’ with the original turbines; and that the overall visual effect will become ‘not significant, neutral’.

Fig. 01. View from A71 of Phase 2 turbines with original turbines in the background.

Source: author’s private photograph.

Other views, such as shown in (Fig.02), were also similarly assessed.

Fig. 02. View from Hartwood of Phase 2 turbines with original turbines in the background.

Source: author’s private photograph.

These visual assessments were qualitative judgements based on the existing procedures for visual assessment, enshrined in the UK. You may agree with those assessments. Interestingly, the landscape architect acting for the local Council, who were the principal objectors at the Inquiry, took a directly opposing opinion. His consideration was that the views contained wind turbines that were ‘out of scale’. The Inquiry was further presented with quantitative evidence, (Fig.03), to illustrate that the scale comparisons between the existing and proposed turbines were up to 2.53 times higher.

Fig. 03. Inquiry diagram proving actual scale comparison.

Source: author’s sketch.

Throughout the developer’s evidence, scale comparisons of up to 2.53 times higher structures were variously dismissed as having: ‘visual and contextual integration’, ‘scale similarity’, ‘adjacent turbine size is linked by perceived scale’, ‘not overbearing or overwhelming’. There was a great leap of judgement from the visualisations presented to the subjective conclusions. It is difficult to imagine how such conclusions were reached. However, reached they were; and sitting within the framework of a full visual study with extensive visual evidence by way of photomontages and the under girding of having followed industry standards and guidelines it was extremely challenging to persuade the Reporter to the Public Inquiry that they appeared to be anything other than acceptable in visual impact terms.

The whole experience of the Inquiry process and the subsequent retrospective study on development completion has prompted this paper. (Fig.04) shows the current UK visual assessment procedure with a host of personal judgement stages crowded at the end of the assessment process. Final decisions on visual acceptability are therefore left to individual professional judgement and any prior objective visual interrogation seems to be limited to inter-visibility, the type of viewer and the location of the view.

Fig. 04. UK guidelines for Visual Assessment with judgment stages circled red by author.

Source: Guidelines for Landscape and Visual Impact Assessment.

These factors give rise to concerns that the process lacks both adequate objective built-in visual assessment techniques and a common visual terminology. It has led me to re-examine how we understand and engage with the images of visualisation and what meaning we place on them. In the next section, I want to explore how it is possible to engage more fully with some basic visual characteristics to inform professional judgement and outline a staged process of visual perception including detailed terminologies to establish a better vocabulary in visual assessment studies.

Thinking

Visual characteristics or descriptors were first assigned by Aylward and Turnbull to visual assessment work in 1978. These descriptors formed the basis for more detailed visual examination and modelling at the time, but fell out of favour as more realistic visualisations became the norm during the 1990s and 2000s. In some ways, this has led to a lazy approach to visual assessment and an increasing reliance on simple professional judgement of images and photomontages. There is a need to improve our understanding of what we are looking at and how we associate meaning to what we see. A framework for visual perception and a progressive revelation of meaning is illustrated in (Fig.05).

Fig. 05. The process of Visual Perception in architectural design.

Source: author’s sketch.

The various stages are noted as:

Stage 1: Preliminary Assessment: Establish the scale of the problem

- Visibility. Determining whether or not it is physically possible to see the object. This is commonly understood as inter-visibility.

- Detectability.  Is there any stimulus pattern from the object present in the viewer’s field of view, i.e. can the viewer detect a change in the visual scene? (Author looking at possible components called visual trend, visual redundancy and visual acuity (using % steradians in the visual field of view) in this section.)

Stage 2: Detailed Assessment: Determine the complexity of the problem

- Discrimination. Can the object be discriminated from its background or adjacent surfaces in terms of its brightness or figural unity? (Author looking at possible components called visual absorption and visual patterning in this section.)

- Legibility. Can the stimulus pattern of the object be read and understood? This task requires the observer to seek some meaning in the object. (Author looking at a possible component called visual definition in this section.)

- Identification. The viewer seeks to bring detailed meaning to the object by identifying critical features and attributes, e.g. edge.

- Recognition. The coalescence of discrimination, legibility and identification whereby the viewer considers whether the object is recognisable from past experience. The introduction of something new to a scene creates visual tension and completes recognition. Here, the designer can fully understand the image and move towards the next stage, judgement. (Author looking at a possible component called visual scaling in this section.)

Stage 3: Design Revision: Decide the solution to the problem

- Judgement. Having recognised or understood the object, the viewer is now required to exercise some critical assessment of its visual qualities and character in the environment. (Author looking at possible components called visual capacity and visual saturation in this section.)

- Manipulation. Within the context of architectural design, this stage permits the designer to adjust the features or character of the object to improve visual acceptability.

- Modification. Within the context of architectural design, this stage permits the designer to modify the form of the design to improve visual acceptability.

These components of visual perception present the process of visual assimilation in a more engaging and meaningful way, seeking tentatively to explore those objective visual elements that may help to unlock the basis of subjective judgements. (Fig.06) shows a visual assessment taxonomy for the critical stages of discrimination, legibility and identification. The visual descriptors in the matrix are the informants to the task of recognition.

Fig. 06. Visual assessment taxonomy in architectural design.

Source: author’s sketch.

The author has devised simple mathematical routines for the assessment of some of the visual descriptors outlined in (Fig.06) and these are the subject of ongoing case study applications to explore the potential of quantitative measures placing meaning on visualisations while guiding and informing those making qualitative visual judgements.

Conclusion

Current industry guidance in the field of visual analysis has been well developed over recent decades. However, despite a plethora of techniques and methodologies, it is still too common for diametrically opposing views to be held by participants at Public Inquiries. This paper has attempted to identify the framework to assist a deeper understanding of how and what we see, and how we might associate meaning to it. In exploring some of the key aspects of visual perception the aim is to help re-establish a basic and commonly accepted visual vocabulary.

The future will be to continue to unlock quantitative aspects of our visual experience to inform and correlate with our qualitative judgements; that may be a long way off, but the challenge is to use digital visual analysis to assemble the components of a quantitative assessment process. It is more than likely that we will never find a common mechanism for interpreting or associating meaning to images, but that does not preclude an attempt to start the journey.

Bibliography

Books:

Landscape Institute and Institute of Environmental Management and Assessment. Guidelines for Landscape and Visual Impact Assessment. 3rd Edition. Oxon: Routledge. 2013.

Purdie, Cameron. Computer Aided Visual Impact Analysis. PhD Thesis. University of Strathclyde. 1983.

Publications and documents without author:

Scottish Power Renewables. Black Law Windfarm Extension Phase 2 Addendum. Glasgow. 2012.

Bayou Bluenvironment. Report on behalf of North Lanarkshire Council on the Landscape and Visual Impacts of the Proposed Black Law Wind Farm Extension Phase 2. Newark. 2013.

Paper in journal:

Aylward, Graeme and Turnbull, Mark. ‘Visual Descriptors’; Design Methods and Theories Journal, vol 12 No 2; 1978.

Scottish Power Renewables. Black Law Windfarm Extension Phase 2 Addendum. Chapter A6, page 5.

Landscape Institute and Institute of Environmental Management and Assessment. Guidelines for Landscape and Visual Impact Assessment. 3rd Edition.

Bayou Bluenvironment. Report on behalf of North Lanarkshire Council on the Landscape and Visual Impacts of the Proposed Black Law Wind Farm Extension Phase 2. Pages 15 and 16.

Scottish Power Renewables. Black Law Windfarm Extension Phase 2 Addendum. Chapter A6, pages 1 to 9.

Landscape Institute and Institute of Environmental Management and Assessment. Guidelines for Landscape and Visual Impact Assessment. 3rd Edition, page 99.

Aylward, Graeme and Turnbull, Mark. ‘Visual Descriptors’; Design Methods and Theories Journal